Product specification

| Customer |  | Annella | Customer Model | TN-1S8AT |
| --- | --- | --- | --- | --- |
| Product Name |  | 1S 4A | Project Name | TN-1S8AT |
| Main configuration |  | 3.7v NMC Li-ion | Customer Material NO. |  |
| Controlled Revision |  | V0 | Issued Date | 2024/2/23 |
| YUN YI |  |  |  |  |
|  |  |  | Client approval： □OK □NG |  |
| Prepared by/Date | 2024/2/23 |  | Checked by/ Date |  |
| QA Checked by/ Date |  |  | QA Checked by/ Date |  |
| Approved by / Date |  |  | Approved by / Date |  |

1.History of revisions

| Edition | Description | Prepared by | Date | Remarks |
| --- | --- | --- | --- | --- |
| V0 | New | Lu Young | 2024/2/23 |  |

2. Product Specifications

2.1 Sphere of application

This specification is applicable to the 1-cell lithium-ion battery protection board developed and manufactured by SHEN ZHEN YUN YI TECHNOLOGY CO.,LTD，model TN-1S8AT

2.2 Environmental request

  - Functional description

1）  Over-charge voltage protection

2）  Over-discharge voltage protection

3）  Over current protection

4）  Short circuit protection

2.4 Mechanical characteristics

1）  PCM size：L 50(±0.15mm)×W 7(±0.1mm)×T 3.5 mm(MAX)

2）  PCB MATERIA/PCB: L：FR-4, 1.5 oz,1.0±0.1mm

3）  LAYER：2Layers

4）  Plating Method:：HASL-LF

5）  PSR INK：Green,

6)   SILK INK：White

2.5 Connecting description

| Symbol | Description | Symbol | Description |
| --- | --- | --- | --- |
| P+ | Battery output/charging positive pole | B+ | Cell positive pole |
| P- | Battery output/charging negative pole | B- | Cell negative pole |

2.6 Electrical characteristic                                                 (Ta=25℃)

| Contents | Min. | Type | Max. | Tolerance | Unit |
| --- | --- | --- | --- | --- | --- |
| Absolute Maximum Rating |  |  |  |  |  |
| Input Charging Voltage |  | 4.20 |  |  | V |
| Input Charging Current |  |  | 3 |  | A |
| Output Discharging Voltage | 2.90 | 3.70 | 4.28 |  | V |
| Continuous Output Discharging Current |  |  | 4 |  | A |
| Ambient Condition |  |  |  |  |  |
| Operating Temperature | -20 |  | +65 |  | ℃ |
| Humidity (No Water-Drop) | 0% |  | 80% |  | RH |
| PCM Storage Condition/PCM |  |  |  |  |  |
| PCM Storage Temperature PCM | -55 |  | +125 |  | ℃ |
| Humidity (No Water-Drop) | 45% |  | 85% |  | RH |
| Protection Parameters |  |  |  |  |  |
| Over-Charge Voltage Protection (OVP) | 4255 | 4280 | 4305 | ±25 | mV |
| Over-Charge Voltage Protection Release | 4030 | 4080 | 4130 | ±50 | mV |
| Over-Charge Voltage Protection Delay Time | 50 | 100 | 150 |  | mS |
| Over-Discharge Voltage Protection (UVP) | 2820 | 2900 | 2980 | ±80 | mV |
| Over-Discharge Voltage Protection Release | 2920 | 3000 | 3080 | ±80 | mV |
| Over-Discharge Voltage Protection Delay Time | 10 | 25 | 40 |  | mS |
| Over-Current Charge Protection Detection Voltage | -60 | -100 | -140 |  | mV |
| Over-Current Charge Protection (OCC) | 4 | 7 | 9 |  | A |
| Over-Current Charge Protection Delay Time | 7 | 12 | 20 |  | mS |
| Over-Current Discharge Protection Detection Voltage | 175 | 200 | 225 |  | mV |
| Over-Current Discharge Protection （OCD） | 10 | 15 | 20 |  | A |
| Over-Current Discharge Protection Delay Time | 5 | 10 | 20 |  | ms |
| Short Circuit ProtectionDetection voltage (SCP) | 550 | 850 | 1150 |  | mV |
| Short Circuit Protection Delay Time | 200 | 300 | 500 |  | uS |
| Short Circuit Protection Release | Remove Load Or Connect Charger |  |  |  |  |
| Current Consumption |  |  |  |  |  |
| Normal Mode |  | 3.0 | 6.0 |  | uA |
| Other Parameters |  |  |  |  |  |
| Impedance （B-&P-） | 5 | 25 | 50 |  | mΩ |
| Impedance （B+&P+） |  |  | 5 |  | mΩ |
| ID Resistor |  | / |  |  | KΩ |
| NTC Resistor |  | / |  |  | KΩ |
| 0V Battery Charge Function | Available |  |  |  |  |
| ESD Protection Function ESD | no |  |  |  |  |

4. SMT Diagram

| Top layer |  |
| --- | --- |

5.PCB Layout

| Top layer |  |
| --- | --- |
| Top overlayer |  |
| Bottom layer |  |
| Bottom overlayer |  |

6.Electrical Schematic

7.Special Requirements

| NO. |  |  |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

8.PCB diagram / PCB

9.Packing、Storage And Transportation

9.1 Packing guide

- 两片拼板背靠背叠整齐                          ②用防静电气泡卷隔开包装

- 装入防静电袋                                 ④用胶布封口，贴标签

⑤连同产品合格证放入外箱内                       ⑥外箱贴标签后封箱

9.2 Storage

9.2.1 Storage Temperature ：23±5℃

9.2.2 Storage Humidity ：45±15% RH

9.2.3 Should pay attention to ESD

9.3 Transportation

9.3.1 Delivery to your storhouse by express or our deliveryman.

9.3.2 Should pay attention to moisture, moisture, avoid extrusion, impact, etc., to prevent damage to the PCM during transportation.

10. Attachment

10.1 Sample test data

| NO. | Test Project | Test standard | Tested Value |  |  |  |  | Judgement |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 | 2 | 3 | 4 | 5 |  |
| 1 | Overcharge protection voltage | 4.28±0.25V |  |  |  |  |  | OK |
| 2 | Overcharge protection delay time | MAX 150ms |  |  |  |  |  | OK |
| 3 | Over discharge protection voltage | 2.90±0.80V |  |  |  |  |  | OK |
| 4 | Over discharge protection delay time | MAX40ms |  |  |  |  |  | OK |
| 5 | Discharge overcurrent protection current | 10-20A |  |  |  |  |  | OK |
| 6 | Over current discharge protection delay time | MAX20ms |  |  |  |  |  | OK |
| 7 | Static current | ≤6.0 uA |  |  |  |  |  | OK |
| 8 | Impedance | ≤50mΩ |  |  |  |  |  | OK |

10.2 Environmental Requirements

The specification subjects to the EU Directive about RoHS 2.0, and the hazardous substance conforms to the following standard.

| Hazardous substance | Standard（mg/KG） | Remarks |
| --- | --- | --- |
| Cd) | <100 |  |
| (Pb) | <1000 |  |
| (Hg) | <1000 |  |
| (Cr6+) | <1000 |  |
| (PBBs) | <1000 |  |
| (PBDEs) | <1000 |  |
| (DBP) | <1000 |  |
| （BBP） | <1000 |  |
| (DIBP) | <1000 |  |
| ）(DEHP) | <1000 |  |

Declaration: the above standard is the requirements of EU RoHS 2.0 Directive, we will base on the customer’s requirements when it is stricter than the EU standard.